HPLC Fingerprint and LC-TOF-MS Analysis on Extract from Roots of Gentiana macrophylla

SU Qi1, SHANG Ping-ping1, ZHANG Yong-min2, JIANa1, HE Jiao1, ZHAO Wen-na1, SUN Wen-ji1*

1. Biomedicine Key Laboratory of Shaanxi Province, Northwest University, Xi’an 710069, China

2. Institut Parisien de Chimie Moléculaire, Université Pierre et Marie Curie-Paris 6, 75005 Paris, France

Abstract:    Objective    Establishing  a  ﬁngerprint  method  to  identify  the  characteristic  chemicals  in  the  roots  of  Gentiana macrophylla  and  evaluate  their  quality.  Methods     RP-HPLC   was   developed   for  ﬁngerprint  analysis  and determination of four ingredients in G. macrophylla roots from different sources. LC-ESI-TOF-MS was employed to identify the chromatographic peaks of the ﬁngerprint. Results    Five common peaks were identified by comparing their retention time with reference secoiridoid glucosides. Eight major peaks in chromatographic fingerprint were analyzed  by  on-line  LC-ESI-TOF-MS.  Four  secoiridoid  glucosides  were  identified  based  on  their  MS  data. Conclusion    The  method  is   specific  and  could  be   served  for  the  quality  identiﬁcation  and  comprehensive evaluation of G. macrophylla.

Key words: fingerprint; Gentiana macrophylla; LC-ESI-TOF-MS; quality control; secoiridoid glucosides

DOI: 10.3969/j.issn.1674-6384.2012.03.012

# Introduction

Gentiana macrophylla Pall. is widely distributed and   used    in    China.    Its    dry    roots    (Gentianae Macrophyllae   Radix,   Qinjiao   in    Chinese)   possess various    therapeutic     activities,    such     as    treating rheumatoid  arthritis,  liver  protection,  and  choleretic effect,  etc  (Pharmacopeia  Committee  of P.  R.  China, 2010).  During  the  past  two  decades,  it  has  been revealed that the roots of G. macrophylla could generate a variety of secoiridiod glycosides, iridoid glycosides, dihydroflavone,   triterpenes,    sterols,    and   inorganic elements, etc (Chen, Shi, and Tu, 2005; Jiang, Liu, and Shang,   2010).   Recently,   HPLC   method   has   been frequently used  in  analysis  on  secoiridoid  glucosides and  related  compounds  in  G.  macrophylla  (Cao  and Wang, 2010a; 2010b; Yu, 2010).

According  to  Chinese  Pharmacopoeia  2010,  G.

macrophylla,  G.  crassicaulis  Duthie.  ex  Burkill,  G. dahurica Fisch. and G. straminea Maxim. were all used as   Gentianae   Macrophyllae   Radix.    However,   the chemical   constituents   of   these    four   species   are significantly different (Wu, Chen, and Yu, 2008; Wang

et  al,  2009;  Cao,  Li,  and  Wang,  2008).  Our  work focused  on  one  of  the  four  species,  G.  macrophylla (Daye Qinjiao in Chinese).

It  is  well  known  that  the  therapeutic  effects  of traditional Chinese medicines (TCM) are based on the synergic effect of their bioactive compounds, which are totally    different    from    that    of    chemical    drugs. Determination    of    a     few     components     is     not representative adequately (Kong, Zhao, and Xiao, 2009; Di, Chan, and Leung, 2003). Taking G. macrophylla as an  example, the  specified  quality  control  method  for detecting   the   total    content   of   loganic    acid   and gentiopicroside in G. macrophylla could not completely reflect  its  quality  (Pharmacopeia  Committee  of P.  R. China, 2010). Moreover, the clarification of the main constituents lacks confirmation based on the retention time. Therefore, it is necessary to establish an effective method   to   clarify   the   main   constituents   of   G. macrophylla and control the quality.

The HPLC ﬁngerprint technique has been considered to  be  a  useful  method  in  identification  and  quality evaluation of herbs and their related ﬁnished products

* Corresponding author: Sun WJ    E-mail: cxbml@nwu.edu.cn

Received: October 26, 2011; Revised: December 14, 2011; Accepted: January 18, 2012

Fund: National Science and Technology Ministry “Significant new drugs creation” special science and Technology Major (20082X09101-016)

in  recent  years,   since  it   could   systematically   and  comprehensively exhibit the types and quantification of  the components in the herbal medicines (Wang et al,  2007; 2006). By now, HPLC coupled with electrospray  ionization  tandem  mass   spectrometry  (ESI-MS)  has  become   a   powerful   technique   for   analysis   and  identification of the chemical constituents in complex  TCM systems (Kang et al, 2008; Han, Shen, and Chen,  2008; Guo et al, 2007; Yang et al, 2007; Liu et al, 2007; Han et al, 2007).

In  previous   studies,  HPLC   fingerprints   of  G.

crassicaulis  (Wang  et  al,  2009),  and  G.  macrophylla (Cao, Li, and Wang, 2008; Ma, Deng, and Yang, 2010) were established, but they were deficient in objectives and comprehensive evaluations. The present study was carried out to explore a complete method which could be well utilized to control the quality of the roots of G. macrophylla.    The    chemical    constituents    in    the ﬁngerprint     were      identiﬁed     based     on     liquid chromatography-electrospray   ionization-time-of-ﬂight mass  spectrometry  (LC-ESI-TOF-MS).  Moreover, the

contents  of the  main  constituents  in  the  roots  of  G.

macrophylla were quantified by HPLC-UV, including loganic    acid,     swertiamarin,    gentiopicroside,    and sweroside.

# Materials and methods

Instruments and chromatographic methods

The  HPLC  system  consisted  of  a  Waters  2695 Separation  Module  (USA)  and  a  Waters  2487  Dual Absorbance  Detector  (USA).  The  output  signal  was recorded using Empower Data Acquisition and Analysis System. Chromatographic separation was achieved on a Welchrom-C18 column (250 mm × 4.6 mm, 5 μm).

HPLC analysis    The mobile phase consisted of CH3OH   (25%)   and   H2O   acidulated   with   0.1% phosphoric acid (75%). The flow rate was 1.0 mL/min and  column  temperature  was  30  ℃.  UV  detection wavelength   was    set    at    236    nm    for    acquiring chromatograms.

LC-ESI-TOF-MS  analysis    The  mobile  phase consisted of CH3OH (A) and 0.1% aqueous formic acid (B). The elution program was as follows: 0－28 min, 31% A; 28－29 min, 31%－37% A; 29－40 min, 37% A;   The   flow   rate   was   0.3   mL/min   and   column temperature was 30 ℃. UV detection wavelength was

set at 236 nm for acquiring chromatograms.

The MS acquisition parameters were as follows: source type ESI, focus not active, scan from m/z 50 to 3000 m/z, ion polarity negative, set capillary 3500 V, dry  heater  180  ℃ and  dry  gas  8.1  L/min.  All  data acquired  were  processed  by  Bruker  Compass  Data Analysis 4.0 software.

Solvents and reagents

CH3OH (HPLC grade) was purchased from Baker Company (USA). Ultra high purity water was prepared by a Millipore-Q SAS 67120 Molsheim (France). Other chemicals  were  of  analytical  grade  with  purity  over 99.5% and used without further purification. Sweroside was provided by Shenzhen Medherb Biotechnology Co., Ltd.  Loganic  acid,  swertiamarin,  and  gentiopicroside were isolated from G. macrophylla in our laboratory. The purity of the isolated compounds was shown to be higher than 98% analyzed by HPLC, and their structures were identified by IR, 1H-NMR, and 13C-NMR.

Plant materials

Fourteen  batches  of  Gentiana  macrophylla  Pall.

(No.  1 －14)  were   collected  at  different  ages  from Longxian   of   Shaanxi    Province,   China    and   were identified as the genuine medicinal herb by Professor WANG  Ya-zhou,  Northwest  University.  A  voucher specimen  (1007)  was  deposited  at  Biomedicine  Key Laboratory of Shaanxi Province, Northwest University. Root  powder  (0.5  g)  was  accurately  weighed  and extracted with 20 mL of methanol in ultrasonic bath (500 W, 40 kHz) for 30 min at 30 ℃ and filtered. The continual  filtrate  (1  µL)  was  diluted  to  a   10  mL volumetric  flask  by  water  as  sample  solution  and filtered  through   a  0.22  µm  filter  membrane  before analysis.  Sample  solution  (10  µL)  was  injected  to HPLC  column  and  separated  under  above-mentioned chromatographic conditions.

Preparation of reference solution

Accurately weighed reference substances, loganic acid,   swertiamarin,   gentiopicroside,   and   sweroside, dissolved   in   methanol.   Mixed   reference   substance solutions    contained    loganic    acid    0.081    mg/mL, swertiamarin   0.046    mg/mL,   gentiopicroside    0.410 mg/mL, and sweroside 0.009 mg/mL, and were filtered through a 0.22 µm filter membrane before analysis.

Data analysis

Data   analysis   was   performed   by   professional

software   named   Similarity   Evaluation    System   for  Chromatographic Fingerprint of TCM (Version 2004 A), which  was  developed  and  recommended  by  Chinese  State Food and Drug Administration. In this study, the  software    was    employed    to    synchronously    do  quantitative  comparison  among  different  samples,  as  well    as    to    compute    and    generate    the    mean  chromatogram as a representative standard fingerprint  chromatogram for a group of chromatograms. Then the  correlation    coefficients     of    samples    with     mean  chromatogram could be provided.

Furthermore, the relative retention time (RRT) and relative  peak  area  (RPA)  of  each  characteristic  peak related  to  the  reference  peak  were  calculated  for quantitative expression of the chemical properties in the chromatographic pattern of herbs.

# Results

Selection of chromatographic condition

For the simultaneous determination of these four compounds,  various  mixtures   of  CH3OH-H2O   and CH3CN-H2O were tested as mobile phase. Considering the  presence  of  loganic  acid  in  herbal  extraction,  a small amount of phosphoric acid was added into the mobile phase to reduce ionization of these compounds. The  results  showed  that  the  best  resolution  and  the shortest analysis time were achieved when methanol- water/phosphoric acid (100:0.1) system was used.

The results indicated that 30 ℃ was the optimum column temperature.

Selection of detection wavelength was one of the key factors for simultaneous determination of loganic acid,  swertiamarin,  gentiopicroside,  and  sweroside.  It was     observed     that     the     maximum     absorption wavelengths of these four compounds were 231, 236, 254, and 243 nm, respectively. It was observed that 236 nm  should  be  selected  as  the  detection  wavelength where   the   four   compounds   in   the   chromatogram possessed strong UV absorbance (Fig. 1).

In LC-MS, because of the limitation of flow rate,

in isocratic elution mode, retention time (tR) of some components in the sample was too long, therefore we used  linear  gradient  elution.  Taking  into  account  the residual phosphate in mass spectrometer, we replaced phosphoric   acid   aqueous    solution   to   formic    acid aqueous solution.

3

A

1

2

4

10

0

20

5

15

3

1

B

2

4

0                5                 10                 15               20

t / min

# Fig. 1   HPLC chromatograms of mixed reference substances

# (A) and sample (B)

1: loganic acid    2: swetiamarin    3: gentiopicroside    4: sweroside

Linearity range

The four compounds were demonstrated to have good linear regression with high correlation coefficient values between peak area (Y) and amount (X, μg). All the  analytes   showed  good  linearity  (r2    >  0.999)  in concentration ranges (Table 1).

Precision

Injection  precision  was  assessed  by  repetitive injections ofthe same sample solution for six times in 1

d. The RSD ofRPA was lower than 1.12%.

Repeatability

Repeatability  was  determined  by  analyzing  six independently  prepared  samples  of  G.  macrophylla

# Table 1    Linearity of four ingredients in G. macrophylla (n = 6)

| Compounds | Range / (μg·mL-1) | Regression equations | 2 r |
| --- | --- | --- | --- |
| loganic acid | 81－1296 | Y = 817 021X－23 458 | 0.9994 |
| swertiamarin | 46－736 | Y = 131.04X－24999 | 0.9994 |
| gentiopicroside | 410－6560 | Y = 105.29X－159 296 | 0.9993 |
| sweroside | 8.9－142.4 | Y = 0.014 392X－2534.1 | 0.9995 |

using  the  same  method.  RSD  of RPA  was  less  than 2.22%.

Sample stability

Stability   was   evaluated   by   analysis   of   six injections of the same sample solution every 3 h at 25 ℃. The RSD of RPA was found below  1.79%.  The results indicated that the sample remained stable for 12 h.

Recovery

The  recovery  test  was  carried  out  by  spiking certain  known  quantity  of  the  four  references  with pulverized sample (0.25 g) of G. macrophylla and six samples were prepared for the test simultaneously. The recoveries of the quantitative compounds proved to be at  the  range  from  99.3%  to   103.23%,  and  the  RSD value was less than 2.80%. It is an acceptable result for recovery analysis.

Sample determination

The valid method was employed to determine the four ingredients in each sample of 14 batches, and the

contents of ingredients were shown in Table 2.

# Table 2    Determination  of 14 batches of G. macrophylla

(μg·mL-1)

| Sample No. | Loganic acid | Swertiamarin | Gentiopicroside | Sweroside |
| --- | --- | --- | --- | --- |
| 1 | 3.21 0.43 |  | 5.31 | 0.030 |
| 2 | 1.53 0.58 |  | 7.85 | 0.070 |
| 3 | 1.62 0.55 |  | 7.50 | 0.040 |
| 4 | 0.67 0.37 |  | 4.92 | 0.040 |
| 5 | 4.19 0.42 |  | 5.88 | 0.090 |
| 6 | 1.96 0.33 |  | 5.22 | 0.040 |
| 7 | 1.00 0.37 |  | 7.13 | 0.060 |
| 8 | 1.99 0.43 |  | 6.60 | 0.010 |
| 9 | 3.26 0.28 |  | 4.61 | 0.040 |
| 10 | 0.96 0.53 |  | 9.38 | 0.000 |
| 11 | 0.66 0.48 |  | 9.02 | 0.020 |
| 12 | 0.58 0.46 |  | 8.32 | 0.020 |
| 13 | 0.52 0.51 |  | 9.71 | 0.000 |
| 14 | 0.74 0.49 |  | 9.13 | 0.000 |
| Average | 1.635 0.445 |  | 7.184 | 0.003 |

Comparison  of  contents  of  four   constituents from G. macrophylla at different ages

By   comparing   the   contents   of   loganic   acid, swertiamarin, gentiopicroside, sweroside, and the sum of these four constituents (Table 2, sample No. 2－4), we observed that different harvest time could influence the   constituent    contents   of   G.    macrophylla.   The differences between two-year and three-year were not

obvious, but if harvested in four-year, the contents in most of the components decreased significantly. Thus, the best time of harvest should be two or three years.

The variation of accumulated ingredient contents from G. macrophylla at different ages could be ascribed to  its   stage  of  flowering,   since  the  three-year   G. macrophylla  experienced  the  stage  of  flowering  and fruiting, while the two-year flowering very few (Cao and Wang, 2010b).

Establishment  of chromatographic  fingerprint for cultured G. macrophylla

Selection of the samples    Fourteen batches of G.

macrophylla  samples were analyzed. There were five common  peaks  identified  in  the  fingerprint  of  G. macrophylla   (peaks    1  － 5,   Fig.    2).   Though    the chromatograms   of   14   batches   of   G.   macrophylla samples were generally similar, the peak area ratios of the common peaks or some small peaks were distinctly different.

Selection  of  reference  substance    To  calculate  the  RRT  and  RPA,  a  reference  substance  should  be  chosen  (Kang  et  al,  2008).  There  are  two  kinds  of  reference   substances,   one   is   an   internal   reference  substance belonging to common peaks and the other is  an external reference substance added to the sample. In  this study, peak 5 (tR  = 15.7 min, gentiopicroside, Fig. 2) was chosen as the internal reference substance because  this  peak  had  maximum  content,  and  existed  in  all  chromatograms. The RRT and RPA of common peaks  in simulative mean chromatograms of G. macrophylla  from   different   sources   were   calculated.   The   RRT  averages of peaks 1－5 were 0.380, 0.602, 0.682, 0.771, and  1.000,  respectively;  The  averages  of  RPA  were  0.0174, 0.2124, 0.0385, 0.0776, and 1.000, respectively.

Chromatograms similarities of 14 batches of G.

macrophylla    The  chromatograms  similarities  of  14 batches  of G.  macrophylla were  0.965,  0.999,  1.000, 0.997, 0.945, 0.994, 0.997, 0.995, 0.946, 0.995, 0.993, 0.992, 0.991, and 0.993, respectively.

HPLC-ESI-TOF-MS analysis

HPLC-ESI-TOF-MS was employed to analyze the components   separated   by   HPLC.   In   ESI-TOF-MS experiment, accurate molecular mass of the components was obtained. Comparing MS results with the standard samples of loganic acid, 6,-O-β-D-glu-gentiopicroside, swertiamarin, and gentiopicroside, four compounds were

5

A

2

4

3

S14 S13 S12 S11 S10 S9   S8   S7   S6   S5   S4   S3   S2   S1

1

0         3.77       7.54        11.30      15.07      18.84      22.61     26.38

5

B

2

4

1               3

0         3.77       7.54        11.30      15.07      18.84      22.61     26.38 t / min

# Fig. 2    Overlap of HPLC chromatograms (A) and HPLC

chromatogram (B) for 14 batches of G. macrophylla

1: unknown    2: loganic acid    3: 6′-O-β-D-glu-gentiopicroside

4: swertiamarin    5: gentiopicroside

unambiguously identified based on their MS data. Since  negative ionization ESI mode was used, most of the m/z  data were [M－H]-  and [M + Cl]- . The mass data and  the tentative assignment of the peaks are given in Table 3, TIC TOF-MS ofthe sample and TOF-MS of peaks 1－8  were shown in Fig. 3.

In ESI(-)-TOF-MS data, [M－H]-  is 375.1 ([M - H]-,  calculated  for  C16H24O10   376.3),  so  peak  3  was deduced as loganic acid (Zhang et al, 2003). [M + Cl]- is 553.1 ([M + Cl]-  calculated for C22H30O14 553.4), so peak   5  was  positively   identified   as   6′-O-β-D-glu- gentiopicroside.   [M   +   Cl]-    is   409.1   ([M   +   H]+, calculated for C16H22O10 374.3), so peak 6 was deduced as  swertiamarin  (Aberham  et al, 2011).  [M  +  Cl]-  is 391.1 ([M + Cl]-, calculated for C16H20O9   391.3),  so peak  8  was  deduced  as  gentiopicroside  (Yang  et  al, 2009). Unfortunately, sweroside is not ionizable under the   chosen   MS    conditions,   probably   due   to   its extremely low content.

# Discussion

In this paper, we established an HPLC ﬁngerprint  analysis method to evaluate the quality of the roots of G. macrophylla, the method is faster and more convenient

# Table 3    Mass data and compound names of peaks

| Peak | t / min | Observed mass | Calculated mass | Assignment |
| --- | --- | --- | --- | --- |
| 1 | 8.5 | 539.1 |  | unknown |
| 2 | 9.0 | 683.2 |  | unknown |
| 3 | 15.5 | 537.2 |  | unknown |
| 4 | 19.2 | 375.1 [M－H]- | 375.3 [M－H]- | loganic acid |
| 5 | 20.9 | 553.1 [M + Cl]- | 553.4 [M + Cl]- | 6′-O-β-D-glu-gentiopicroside |
| 6 | 24.4 | 409.1 [M + Cl]- | 409.3 [M + Cl]- | swertiamarin |
| 7 | 27.7 | 389.1 |  | unknown |
| 8 | 30.5 | 391.1 [M + Cl]- | 391.3 [M + Cl]- | gentiopicroside |

than previous reports (Wang et al, 2009; Cao, Li, and Wang, 2008). The contents ofthe four compounds were determined. Meanwhile, as one of the most powerful analytical techniques, LC-ESI-TOF-MS was utilized to confirm the  structures  of the  main  constituents  in  G. macrophylla, which provided a high level of sensitivity and  resolution  (Thurman,  Ferrer,  and  Zweigenbaum, 2005; Han, Shen, and Chen, 2008).

An  HPLC  method  was   developed  to  obtain  a fingerprint   for   the   roots   of   G.   macrophylla.   The fingerprint showing five common peaks represented the characteristics of constituents in G. macrophylla.

The similarities of 14 batches of G. macrophylla  samples were obtained with a standardized procedure.  In  order  to  further  adequately  and  comprehensively  assess  the  quality  of  G.  macrophylla  probably,  more  than  one  fingerprint  would  be  needed  due  to  the  complexity of the constituents in herbal drugs. However, it  could  be  acceptable  as  a  first  step  to  create  a  representative fingerprint, as presented here, and more  fingerprints as new evidence will be studied later. In  addition, by comparing the main component contents of

G. macrophylla at different ages, two-year G. macrophylla showed the highest content. Thus, two or three years

A

4

6

5

3

25

0             5             10           15           20

8                                                 1   2

B

4

5

3

15

6

7

35

0

10

20

25

5

30

8

7

30

35

t / min

C

200        400        600          800         1000       1200       1400                        200         400         600          800         1000       1200       1400

200        400        600          800         1000       1200       1400                        200         400         600          800         1000       1200       1400

200        400        600          800         1000       1200       1400                        200         400         600          800         1000       1200       1400

200        400        600          800         1000       1200       1400                        200         400         600          800         1000       1200       1400 m / z

# Fig. 3    HPLC chromatograms (A), TIC TOF-MS of sample 9 (B), and TOF-MS of peaks 1－8 (C)

1－3 and 7: unknown    4: loganic acid    5: 6,-O-β-D-glu-gentiopicroside     6: swertiamarin    8: gentiopicroside

should be the best time of harvest. To our knowledge, this is the first report on the analysis of G. macrophylla by LC-ESI-TOF-MS technique.  The results  indicated that this technique would be useful in comprehensive quality evaluation of G. macrophylla.

References

Aberham  A,  Pieri  V,  Croom  EM,  Stuppner  H,  2011.  Analysis  of iridoids  secoiridoids  and  xanthones  in   Centaurium  erythraea, Frasera  caroliniensis  and  Gentiana  lutea  using  LC-MS  and RP-HPLC. JPharm BiomedAnal 53(3): 517-525.

Cao  XY,  Li  YH,  Wang  ZZ,  2008.  Study  on  HPLC  fingerprint  of

Gentiana macrophylla. J Chin Med Mater 31(4): 498-501.

Cao  XY,  Wang  ZZ,  2010a.  Simultaneous  determination  of  four iridoid  and  secoiridoid  glycosides  and  comparative  analysis  of Radix  Gentianae Macrophyllae  and  their related  substitutes by HPLC. PhytochemAnal 21: 348-354.

Cao XY, Wang ZZ, 2010b. Determination of four secoirid glucosides

in   Gentiana   crassicaulis    during   different    harvesy   periods cultivated in Yunnan province. Chin JPharm Anal 30: 623-625.

Chen QL, Shi ZY, Tu GZ, 2005. Studies on the chemical constituents in  roots  of  Gentiana  macrophylla  from  Shaanxi.  Chin  J  Chin MaterMed 30: 1519-1522.

Di X,  Chan  KCC,  Leung  HW,  2003.  Fingerprint profiling  of acid hydrolyzates of polysaccharides extracted from the fruiting bodies and    spores     of    Lingzhi    by    high-performance    thin-layer chromatography. J ChromatogrA 1018: 85-95.

Guo H, Liu AH, Ye M, Yang M, Guo DA, 2007. Characterization of phenolic  compounds  in  the  fruits  of  Forsythia  suspensa   by  high-performance     liquid      chromatography      coupled     with electrospray ionization tandem mass spectrometry rapid commun. Mass Spect 21: 715-729.

Han   C,   Shen   Y,    Chen   JH,   2008.   HPLC    fingerprinting   and LC-TOF-MS    analysis    of    the    extract    of    Pseudostellaria heterophylla (Miq.) Pax roots. J Chromatogr B 862: 125-131.

Han   J,   Ye   M,   Xu   M,   Sun   JH,   Wang   BR,   Guo   DA,   2007.

Characterization  of flavonoids  in the traditional  Chinese herbal

medicine   Huangqin  by   liquid   chromatography   coupled  with electrospray ionization mass spectrometry. J Chromatogr B 848: 355-362.

Jiang ZB, Liu HL, Shang JN, 2010. Chemical constituent of Radix

Gentianae Macrophyllae. Nat Prod Res 24: 1365-1369.

Kang   J,   Zhou   L,   Sun   JH,   Guo   DA,   2008.   Chromatographic fingerprint analysis and characterization of furocoumarins in the roots of Angelica dahurica by HPLC/DAD/ESI-MSn  technique. J Pharm BiomedAnal 47: 778-785.

Kong  WJ,  Zhao  YL,  Xiao  XH,  2009.  Quantitative  and  chemical fingerprint  analysis  for  quality  control  of  Rhizoma   Coptidis Chinensis  based  on  UPLC-PAD  combined  with  chemometrics methods. Phytomedicine 16: 950-959.

Liu AH, Lin YH, Yang M, Guo H, Guan SH, Sun JH, Guo DA, 2007.

Development  of the  fingerprints  for the  quality  of the roots  of Salvia  miltiorrhiza  and  its related preparations by  HPLC-DAD and LC-MSn. J Chromatogr B 846: 32-41.

Ma  XQ,  Deng   WF,  Yang   DH,  2010.  HPLC   fingerprinting  and

quantification of gentiopicroside and loganic acid in  Gentianae

Macrophyllae Radix crude drugs. J Chin Pharm Sci 19: 243-250.

Pharmacopeia  Committee  of P. R.  China, 2010. Pharmacopoeia of People,s   Republic   of    China.   China    Medical   Science    and Technology Press: Beijing.

Thurman   EM,   Ferrer   I,   Zweigenbaum   JA,   2005.   Discovering metabolites   of  post-harvest   fungicides   in   citrus   with   liquid chromatography/time-of-flight  mass  spectrometry  and  ion  trap

tandem mass spectrometry. J ChromatogrA 1082: 71-80.

Wang TT, Chen XH, Hu QQ, Bi KS, 2006. RP-HPLC fingerprint for quality  assessment  of Radix Angelicae Dahuricae. Acta Pharm Sin 41: 747-751.

Wang TT, Jin H, Li Q, Cheng WM, Hu QQ, Chen XH, Bi KS, 2007.

Isolation and simultaneous determination of coumarin compounds in Radix Angelica Dahurica. Chromatographia 65: 477-481.

Wang YY, Zhao ZL, Wu JR, Wang ZT, 2009. HPLC fingerprint of Radix  Gentianae  Crassicaulis.  Chin  Tradit  Herb  Drugs  40(1): 120-123.

Wu YX, Chen G, Yu CY, 2008. Chemical constituents of Gentiana

straminea Maxim. J Beijing Univ Chem Tech 35(2): 64-67.

Yang  L,  Wang  Y,  Wang  LX,  Xiao  HB,  Wang  ZT,  2009.  Rapid quantification of iridoid glycosides analogues in the formulated Chinese   medicine   Longdan   Xiegan   Decoction   using   high- performance    liquid     chromatography     coupled    with     mass spectromentry. J Chromatogr A 1216(11): 2098-2103.

Yang M, Wang XM, Guan SH, Xia JM, Sun JH, Guo H, Guo DA, 2007.  Analysis  of  triterpenoids  in   Ganoderma  lucidum  using liquid chromatography coupled with electrospray ionization mass spectrometry. JAm Soc Mass Spectrom 18: 927-939.

Yu  QF,  2010.  Compare  with  the  active  constituents  in  different  districts of Radix Gentianae Macrophyllae. J Jiangxi Norm Univy: Nat Sci 34: 174-177.

Zhang XF, Xu Q, Xiao HB, Liang XM, 2003. Iridoid glucosides from

Strychnos nuxvomica. Phytochemistry 64(8): 1341-1344.